Release Note –Donseed Scan v2.4.4

Bluebox 4G Clocks

 During sync process when Donseed scan request WMS server for

templates for New unit or force synced units

Issue: if unit has low internet signals the download sync process is interrupted

Reason: There are two reasons for interruption

 Permanent loss of internet

Fix: Stores a template id and index. Next time when starts downloading templates sync process will check if any stored id. Donseed scan will pick up and update the server last sync date so server will other gives templates than stored

 Temporary loss of internet

Fix: Once process is interrupted Donseed scan will try to update server last sync date right away if it fails then go to step a.

 Donseed scan reads 4G Clocks from the device in batches

Issue: While reading templates from 4G clocks raise an error

Reason: Large number of templates on 4G clocks will interrupt readers

Fix: Read templates from readers in batches and built a list to process in memory. Set as configuration

Sigma 5G

 Add users to Sigma Clocks

Issue: Add users more than 300 to the device raises an error sometimes but some users are stored to the device

Fix: Adding users to device in batches of 200. Set as configuration

 Remove users from Sigma Clocks

Issue: remove users more than 300 to the device raises an error sometimes but some users are stored to the device

Fix: Removing users from device in batches of 200. Set as configuration

 Force sync remove reader templates

Issue: When force sync reader templates must go through normal template removal process

Reason: Takes too long and sometimes comes up with errors

Fix: Found quick database erase for 4G and 5G clocks

See batch process setting for Donseed Scan 2.4.4

<!-- Batch processing for clock -->

<add key="donseedscan.AddUsersInBatchSigma5G" value="100" />

<add key="donseedscan.AddUsersInBatch4GClock" value="3000" />

<add key="donseedscan.RemoveUsersInBatchSigma5GClock" value="100" />

How to Document for Donseed scan sync templates

New Sigma or 4G Clock

- Add reader in wms

- Assign reader to the site

- Create a sync group

- Add reader to the sync group

Rented Clock/Failed Clock

- Clear reader templates before re-assigning to a new site

- Clear export folder from the pc if the UNIT ID has been assigned before to this pc

Swap Clock in different sync group

- Remove export folder and remove reader templates

- Use force sync from WMS

Conor Bronson Test Findings

There is now a feature within the ‘Duplicate Test App’ (which is accessible through remote log in to the server unit) where the following steps can be carried out:

- Enter organization ID for a particular client which identifies the number of templates they have in total, and separately identifies the number of those templates that have not been used in the last number of selected days. (i.e. 90 days)

- There is then the option to Deactivate the templates of employees who have not been on a timesheet for over the selected number of days.

- When the Donseed unit syncs, all deactivated templates will delete and will delete from other units on the sync group (this is still to be tested).

- If one of the employees who are deactivated comes back again in the future, their profile can be re-activated again within WMS and they can be re-enrolled on the Unit again.