SecureAdmin Certificate Update

User Guide

Table of Contents

DOCUMENT DETAILS	3

Document History	3

References	3

1.	Introduction	4

2.	RECOMMENDED solution: Disable the SSL communication before certificate expiry (i.e. before 15 March, 2020)	5

Using SecureAdmin (preferred option):	5

Using LCD Menu (alternate option)	6

4G/4G Lite VStation series and 3D Face readers:	6

Using LCD on MA SIGMA series (BioScrypt legacy mode):	6

3.	Error messages that may indicate SecureAdmin certificates are expired	7

4.	Steps to update the SecureAdmin Certificate using “Certificate Updater” tool:	8

5.	Steps to disable the SSL communication for all the terminals after the Certificate is expired and SecureAdmin is not able to communicate under SSL connections (after March 15th 2020):	10

Method-1: Using LCD for 4G/4G Lite VStation series, 3D Face readers and SIGMA BioScrypt:	10

Method-2: Using USB communication for all the 4G series and 3DFace readers:	10

Method-3: Using USB script for MA SIGMA Family terminals in BioScrypt Legacy mode:	14

6.	How to Restore the SecureAdmin database if customer loses the data	15

7.	Procedure to login to SecureAdmin or terminal LCD menu if customer lost the password	16

8.	SecureSDK commands to enable / disable SSL communication with terminal	17

9.	Procedure to resolve the error occurred during certificate installation on Windows 7 OS (64 bit, 32 bit)	18

# DOCUMENT DETAILS

## Document History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 26/02/2020 | IDEMIA | First Release |

## References

# Introduction

Legacy Bioscrypt 4G range has been announced end of life in November 2016 and IDEMIA has recommended customers to anticipate a migration to the new generation.

Anyway, some legacy installations certainly remain in operation and it is important for IDEMIA to make sure that they are still running.

IDEMIA has very recently discovered that certificates used for secure connection expire on March 15th, 2020.

The main impacts of this license expiration (if no action is done) are the following:

- Terminals will continue to work and communicate through Wiegand.

- The communication channel between the terminals, server and client will not be working anymore (no data can be updated or retrieved from terminals). As an example, logs related to time and attendance or access control can’t be retrieved, no user can be added or removed

This document describes procedure to upgrade certificates. It also explains the procedure to disable SSL communication between SecureAdmin and reader before and after certificate expiry for 4G/4G Lite family, 3D Face terminals and MorphoAccess SIGMA Family terminals running in BioScrypt Legacy mode.

Note1: This document is applicable only to users who:

  - did chose to keep their legacy system or didn’t get the time to finalize their migration from BioScrypt (devices and SecureAdmin Software) to new SIGMA / MorphoManager

  - use the option to activate TLS communication option between terminals, server and client

Note 2: If TLS/SSL communication between SecureAdmin and reader is enabled, it is advisable to disable it before updating the certificates.

# RECOMMENDED solution: Disable the SSL communication before certificate expiry (i.e. before 15 March, 2020)

## Using SecureAdmin (preferred option):

- Go to  SA->Device Settings->(Select the device from the Device group) -> Communication->Enable SSL communication(check box for Disable/Enable)-> Apply

Secure Admin

- Go to  SALite->Network Tree->Device Network->(Select the device from the Device group) -> Communication->Enable SSL communication(check box for Disable/Enable)-> Apply

Secure Admin Lite

- This procedure to disable the SSL communication is same for all the readers such as 3D Face Reader, 4G family readers and MA SIGMA Series readers running in BioScrypt legacy mode.

## Using LCD Menu (alternate option)

### 4G/4G Lite VStation series and 3D Face readers:

  - Login the LCD menu with default (0000 for 4G series)/user defined password.

  - Go to Security -> Communication->Secure Connection Mode

  - A popup will be observed on LCD with message “Disable Secure Connection Mode?” with option ESC or OK.

  - User can click on OK to disable.

  - The message “Secure Connection disabled” will be displayed

### Using LCD on MA SIGMA series (BioScrypt legacy mode):

  - Login to the LCD menu with default password (12345)/user defined password and Select SSL Configuration under Communication – Security Menu

  - Go to Security Menu –> Communication –> SSL Configuration

  - Device allow user to enable/disable SSL Communication.

# Error messages that may indicate SecureAdmin certificates are expired

Following are the scenarios which would result in error caused because of SecureAdmin certificates expiration:

- If the customer login to SecureAdmin and the new certificates are not updated on SecureAdmin client and server

- If the new certificates are updated on SecureAdmin client/s only and not on the server

- If the new certificates are updated on SecureAdmin server only and not on the client/s

- If the secure communication fails with the terminal when the secure communication between SecureAdmin and reader was enabled

# Steps to update the SecureAdmin Certificate using “Certificate Updater” tool:

This update requires:

- A utility package delivered by IDEMIA support group.

- Administrator rights on the computer

- Extract the archive containing the certificate update tool

- Locate “CertificateUpdater.exe” inside the folder

- Right click on that file and choose option “Run as administrator”

- Click on “Update Certificate” button.

- This will update secure admin certificates

- On successful update of certificates, message “Certificate updated successfully” would appear on the screen

- If user does not run the utility with administrator rights then following error will be displayed:

Note: It is mandatory to run “Certificate Updater” tool on all the PCs running SecureAdmin software as a Client or Server.

- If SecureAdmin client and server both are running on same machine then this tool needs to run only once.

- If SecureAdmin client and server are running on different PCs, then it is mandatory to run this utility on both PCs

# Steps to disable the SSL communication for all the terminals after the Certificate is expired and SecureAdmin is not able to communicate under SSL connections (after March 15th 2020):

## Method-1: Using LCD for 4G/4G Lite VStation series, 3D Face readers and SIGMA BioScrypt:

- Please refer to section to disable SSL Using LCD Menu above as the connection with Secure Admin is not possible any more.

## Method-2: Using USB communication for all the 4G series and 3DFace readers:

- Install SecureAdmin on a PC and directly connect to the terminal using a USB cable – the latest version is available on www.biometric-terminals.com

- Register the 4G terminal (i.e. 4G VStation/3DFace/4G Lite series) to the SecureAdmin. Example of registration via Server is illustrated below (alternately, the registration can be done via Client)

REGISTER VIA SERVER DIALOG BOX

    - Select Serial communication

    - Enter the appropriate Port, Baud Rate, and Device ID for USB/RS-232/RS-485 communication.

    - Check Auto check box if Device ID is not known.

Note:

- To determine the correct port number while device connected on USB, look in the Windows Device Managed for a “Gadget Serial” entry in case of Finger Print and Finger Vein Devices while look for “USB serial” entry for Face Reader Device under the Ports(COM  & LPT) heading.

- When the device is connected to a PC workstation for the first time, a Found New Hardware message will be displayed. The Hardware Installation Wizard prompts you to connect to the internet to search for a driver. Select Not at this time > Next. Then choose Install the driver automatically

- Register the terminal in SA via serial communication as explained above and then disable the SSL communication using the path

  - Go to  SA->Device Settings->(Select the device from the Device group) -> Communication->Enable SSL communication(check box for Disable/Enable)-> Apply

  - Go to  SALite->Network Tree->Device Network->(Select the device from the Device group) -> Communication->Enable SSL communication(check box for Disable/Enable)-> Apply

- Following is the example for 3DFace readers connection using USB/RS-485/RS-232:

USB communication

RS-485 Communication

RS-232 communication

## Method-3: Using USB script for MA SIGMA Family terminals in BioScrypt Legacy mode:

This update requires the utility package delivered by IDEMIA support group.

- Extract the archive containing the certificate update tool

- Locate the 3 USB script files (usb_script.json, config_param.json and Requestin.data) inside the directory USBScript.

- Copy these files onto a USB flash drive

- Safely remove the flash driver from PC

- Attach the flash drive to reader using USB cable (no password required)

- “USB Connected” message should pop up on terminal screen

- “Please Wait…” message should pop up

- Once script is executed “Script Successfully Executed” message should display

- SSL connection should be disabled now

# How to Restore the SecureAdmin database if customer loses the data

Please refer to SecureAdmin Operator manual -> Chapter 18 “BACKUP AND RESTORE DATABASE” for detailed instructions to backup and restore the database.

The location of the tool to perform the backup is indicated in Chapter 18.

This manual is part of the Software. Please refer to Section 5 above to know more on how to download it from www.biometric-terminals.com.

# Procedure to login to SecureAdmin or terminal LCD menu if customer lost the password

- If user forgot the SercureAdmin password then user have to reinstall the server on the PC, but the configuration (and database) of already installed SecureAdmin will be lost.

- In case user forgot terminal LCD login password (4G series, 3DFace, SIGMA series), then one way to setup a new LCD login password is via registering the terminal to SecureAdmin and then go to

  - SA->Device Settings->Device Group(select device)->Device Security->All user password->Password (for SecureAdmin)

  - SALite->Network->Network tree->Device Network->Device Group(select device)->Device Security->All user password->Password (for SecureAdmin Lite)

- If customer is not able to register Terminal through Ethernet then using USB/Serial they can connect SecureAdmin and follow the same path.

- For more information please refer SecureAdmin operator manual.

Secure Admin

Secure Admin Lite

Please refer to Section 5 above to know more on how to download it from www.biometric-terminals.com.

# SecureSDK commands to enable / disable SSL communication with terminal

Following are the commands to get status, enable and disable the SSL communication with terminal:

- Get SSL Connection Mode 0x0993

  - COMMAND ID: 0x0993

  - Request (from host to device)

    - LENGTH: 4

    - DATA FIELDS: None

- Set SSL Connection Mode 0x085D

  - COMMAND ID: 0x085D

  - Request (from host to device)

    - LENGTH: 5

    - DATA FIELDS:

      - 0- Disable SSL connection (Connection to device is non-secure).

      - 1- Enable SSL connection (Connection to device is secured through SSL channel).

# Procedure to resolve the error occurred during certificate installation on Windows 7 OS (64 bit, 32 bit)

Note: This section is applicable for SecureAdmin installed on Windows-7 OS only if certificates cannot be updated. Please ignore this section if there was no error or if SecureAdmin is installed on any other OS.

SecureAdmin certificates to be installed are signed using SHA2, you may have an error during certificate installation on Windows-7 OS related to driver signature. You will need to update your system in order to be able to install SecureAdmin certificates.

Please refer to below mentioned link for more details on certificate signing:

- https://technet.microsoft.com/en-us/library/security/3033929

Steps to install Microsoft Windows-7 KB update (KB3033929):

- Locate KB update binary inside folder WindowsKBUpdateFiles

- Select KB update binary according to 32-bit(Windows6.1-KB3033929-x86.msu) or 64-bit(Windows6.1-KB3033929-x64.msu) Windows-7 OS

- Double click on to that binary

- Follow the onscreen instructions

- If KB update is already installed on PC, “Security Update for Windows (KB3033929) is already installed on this computer” message should pop-up. Please ignore this message and proceed to update certificates.